CSCN8020-26W-Sec1- Reinforcement Learning Programming

LESLEY KAMAMO

8984971
ASSIGNMENT 1

Github Url:

## Problem 1

The pick-and-place robot arm is modeled as an MDP where the state includes joint angles/velocities and task context (object pose, target pose, gripper/contact state), ensuring the Markov property.

We model the task as an MDP:

SAPRγ

At each discrete control step t(e.g., every 20–50 ms), the agent observes state st, chooses action at, the robot transitions to st+1, and receives reward rt+1

The actions are continuous motor commands (joint torques, plus gripper open/close), matching low-level control for smooth motion. Transitions follow robot dynamics with possible stochasticity from noise and friction. The reward combines a large success reward for placing the object, a per-step time penalty to encourage speed, and smoothness/energy penalties on torque magnitude and torque changes to reduce jerk and wear, optionally shaped by distances to the object/goal to improve learning. A high discount factor (e.g., γ=0.99) values completing the full task sequence.